Main.c

#include "stm32f4xx.h"

#include "./usart/bsp_debug_usart.h"

#include "spi_flash.h"

int main(void)

{

uint32_t i;

uint8_t add[4096]={0x00};

uint32_t data;

Debug_USART_Config();

/* 发送一个字符串 */

Usart_SendString( DEBUG_USART,"这是一个FLASH实验\n");

printf("这是一个FLASH实验\n");

printf("开始初始化SPI\n");

SPI_FLASH_Config();

printf("初始化SPI成功\n");

printf("开始读取设备ID\n");

data = device_id();

printf("设备地址0X%X\n",data);

printf("读取设备ID成功\n");

printf("开始擦除\n");

erase_flash(0X00);

printf("擦除成功\n");

printf("开始擦除后的读取\n");

for(i = 0;i < 4096;i++)

{

printf(" 0X%X\n",add[i]);

}

printf("擦除后的读取成功\n");

while(1)

{

}

}

/*********************************************END OF FILE**********************/

spi_flash.h

#ifndef __SPI_FLASH_H

#define	__SPI_FLASH_H

#include "stm32f4xx.h"

//引脚定义

/*******************************************************/

#define SPI_FLASH                               SPI5

#define SPI_FLASH_CLK                           RCC_APB2Periph_SPI5

#define SPI_FLASH_CE_GPIO_PORT                  GPIOF

#define SPI_FLASH_CE_GPIO_CLK                   RCC_AHB1Periph_GPIOF

#define SPI_FLASH_CE_PIN                        GPIO_Pin_6

#define SPI_FLASH_SCK_GPIO_PORT                 GPIOF

#define SPI_FLASH_SCK_GPIO_CLK                  RCC_AHB1Periph_GPIOF

#define SPI_FLASH_SCK_PIN                       GPIO_Pin_7

#define SPI_FLASH_SCK_AF                        GPIO_AF_SPI5

#define SPI_FLASH_SCK_SOURCE                    GPIO_PinSource7

#define SPI_FLASH_MISO_GPIO_PORT                GPIOF

#define SPI_FLASH_MISO_GPIO_CLK                 RCC_AHB1Periph_GPIOF

#define SPI_FLASH_MISO_PIN                      GPIO_Pin_8

#define SPI_FLASH_MISO_AF                       GPIO_AF_SPI5

#define SPI_FLASH_MISO_SOURCE                   GPIO_PinSource8

#define SPI_FLASH_MOSI_GPIO_PORT                GPIOF

#define SPI_FLASH_MOSI_GPIO_CLK                 RCC_AHB1Periph_GPIOF

#define SPI_FLASH_MOSI_PIN                      GPIO_Pin_9

#define SPI_FLASH_MOSI_AF                       GPIO_AF_SPI5

#define SPI_FLASH_MOSI_SOURCE                   GPIO_PinSource9

#define SPI_FLASH_CS_LOW()  GPIO_ResetBits(SPI_FLASH_CE_GPIO_PORT, SPI_FLASH_CE_PIN)

#define SPI_FLASH_CS_HIGH() GPIO_SetBits(SPI_FLASH_CE_GPIO_PORT, SPI_FLASH_CE_PIN)

#define JEDEC_ID             0X9F

#define DUMMY                0XFF

#define Write_Enable         0X06

#define Sector_Erase         0X20

#define Read_Status_Register  0X05

#define Read_Data            0x03

void SPI_FLASH_Config(void);

uint8_t read_write(uint8_t data);

uint32_t device_id(void);

void erase_flash(uint32_t adder);

void read_flash(uint8_t* pata,uint32_t adder,uint32_t size);

#endif /* __SPI_FLASH_H */

Spi_flash.c

#include "spi_flash.h"

void SPI_FLASH_Config(void)

{

GPIO_InitTypeDef GPIO_InitStructure;

SPI_InitTypeDef SPI_InitStructure;

RCC_AHB1PeriphClockCmd(SPI_FLASH_CE_GPIO_CLK

|SPI_FLASH_SCK_GPIO_CLK

|SPI_FLASH_MISO_GPIO_CLK

|SPI_FLASH_MOSI_GPIO_CLK,ENABLE);

/* 使能 SPI 时钟 */

RCC_APB2PeriphClockCmd(SPI_FLASH_CLK, ENABLE);

/* 连接*/

GPIO_PinAFConfig(SPI_FLASH_SCK_GPIO_PORT,SPI_FLASH_SCK_SOURCE,SPI_FLASH_SCK_AF);

GPIO_PinAFConfig(SPI_FLASH_MISO_GPIO_PORT,SPI_FLASH_MISO_SOURCE,SPI_FLASH_MISO_AF);

GPIO_PinAFConfig(SPI_FLASH_MOSI_GPIO_PORT,SPI_FLASH_MOSI_SOURCE,SPI_FLASH_MOSI_AF);

/* GPIO初始化 */

GPIO_InitStructure.GPIO_OType = GPIO_OType_PP;

GPIO_InitStructure.GPIO_PuPd = GPIO_PuPd_NOPULL;

GPIO_InitStructure.GPIO_Speed = GPIO_Speed_50MHz;

GPIO_InitStructure.GPIO_Mode = GPIO_Mode_AF;

GPIO_InitStructure.GPIO_Pin = SPI_FLASH_SCK_PIN  ;

GPIO_Init(SPI_FLASH_SCK_GPIO_PORT, &GPIO_InitStructure);

GPIO_InitStructure.GPIO_Pin = SPI_FLASH_MISO_PIN;

GPIO_Init(SPI_FLASH_MISO_GPIO_PORT, &GPIO_InitStructure);

GPIO_InitStructure.GPIO_Pin = SPI_FLASH_MOSI_PIN;

GPIO_Init(SPI_FLASH_MOSI_GPIO_PORT, &GPIO_InitStructure);

GPIO_InitStructure.GPIO_OType = GPIO_OType_PP;

GPIO_InitStructure.GPIO_PuPd = GPIO_PuPd_NOPULL;

GPIO_InitStructure.GPIO_Speed = GPIO_Speed_50MHz;

GPIO_InitStructure.GPIO_Mode = GPIO_Mode_OUT;

GPIO_InitStructure.GPIO_Pin = SPI_FLASH_CE_PIN  ;

GPIO_Init(SPI_FLASH_CE_GPIO_PORT, &GPIO_InitStructure);

SPI_FLASH_CS_HIGH();   // 初始化后立即拉高片选

//初始化SPI

SPI_InitStructure.SPI_BaudRatePrescaler = SPI_BaudRatePrescaler_4;

SPI_InitStructure.SPI_CPHA = SPI_CPHA_1Edge;

SPI_InitStructure.SPI_CPOL = SPI_CPOL_Low;

SPI_InitStructure.SPI_CRCPolynomial = 7;

SPI_InitStructure.SPI_DataSize = SPI_DataSize_8b;

SPI_InitStructure.SPI_Direction = SPI_Direction_2Lines_FullDuplex;

SPI_InitStructure.SPI_FirstBit = SPI_FirstBit_MSB;

SPI_InitStructure.SPI_Mode = SPI_Mode_Master;

SPI_InitStructure.SPI_NSS = SPI_NSS_Soft;

SPI_Init(SPI_FLASH,&SPI_InitStructure);

SPI_Cmd(SPI_FLASH,ENABLE);

}

uint8_t read_write(uint8_t data)

{

//等待发送缓冲区为空

while(SPI_GetFlagStatus(SPI_FLASH,SPI_FLAG_TXE) == RESET){}

//发送数据

SPI_I2S_SendData(SPI_FLASH,data);

//接受缓冲区为空

while(SPI_GetFlagStatus(SPI_FLASH,SPI_FLAG_RXNE) == RESET){}

return SPI_I2S_ReceiveData(SPI_FLASH);

}

uint32_t device_id()

{

uint8_t data[3];

SPI_FLASH_CS_LOW();

read_write(JEDEC_ID);

data[0] = read_write(DUMMY);

data[1] = read_write(DUMMY);

data[2] = read_write(DUMMY);

SPI_FLASH_CS_HIGH();

return (data[0] << 16 | data[1] << 8) | data[2];

}

void write_enable_flash(void)

{

SPI_FLASH_CS_LOW();

read_write(Write_Enable);

SPI_FLASH_CS_HIGH();

}

void wait_for_regieter()

{

uint8_t status = 0x01;

while(status & 0x01)

{

SPI_FLASH_CS_LOW();

read_write(Read_Status_Register);

status = read_write(DUMMY);

SPI_FLASH_CS_HIGH();

}

}

void erase_flash(uint32_t adder)

{

write_enable_flash();

SPI_FLASH_CS_LOW();

//发送擦除信号

read_write(Sector_Erase);

read_write((adder >> 16) & 0XFF);

read_write((adder >> 8) & 0XFF);

read_write(adder & 0XFF);

SPI_FLASH_CS_HIGH();

//等待擦除完成

wait_for_regieter();

}

void read_flash(uint8_t* pata,uint32_t adder,uint32_t size)

{

SPI_FLASH_CS_LOW();

read_write(Read_Data);

read_write((adder >> 16) & 0XFF);

read_write((adder >> 8) & 0XFF);

read_write(adder & 0XFF);

while(size-- )

{

*pata = read_write(DUMMY);

pata++;

}

SPI_FLASH_CS_HIGH();

}